WORTH TOWNSHIP, BUTLER COUNTY

SUBDIVISION/LAND DEVELOPMENT INSTRUCTIONS

Please read the following carefully. We hope these “walk-through” instructions will be of help to you.

If you plan to make improvements to your lot, which is used for OTHER THAN A RESIDENTIAL single-family dwelling, duplex, or residential out-building, your improvements are considered “land development.”

(See Land Development definition - Municipalities Planning Code, Article I, Section 107 and Chapter 22, Part3. (PLEASE obtain a SUBDIVISION and LAND DEVELOPMENT APPLICATION from the Township office or from the website.

The Township requires submission of a professional drawing (with an engineer's seal) indicating existing improvements AND all proposed improvements in proper format with your ‘land development application.’ Improvements include, but are not limited to grading, paving, parking areas, curbs, gutters, storm sewers and drains, improvements to existing watercourses, sidewalks, crosswalks, street signs, monuments, water supply facilities, sewage disposal facilities, storm water control facilities, and buildings.

Review Fees and Expenses. The applicant/developer shall reimburse the Township for all fees and expenses incurred by the Township related to a subdivision and/or land development and/or the review of any application under this Chapter, including, but not limited to, the reasonable and necessary charges by  the Township Engineer and/or the Township Solicitor for review and report on an application, subdivision and/or land development. As a condition of acceptance, performance bonds may be required for determined improvements.

I. OBTAIN A SUBDIVISION APPLICATION / LAND DEVELOPMENT APPLICATION.

A. Actual costs of review by the Township Engineering Firm or other professional and/or technical assistance and studies determined necessary by the Township Planning Commission and/or Township Board of Supervisors are the responsibility of the applicant. (You will be invoiced by the Township; please remit to the Township.)

B. Complete AND SIGN the appropriate application and submit it with all the information/material required 2 WEEKS prior to the Township Planning Commission meeting.

II. On-Site Disposal System

A. Tests to be performed by the Township Sewage Enforcement Officer

1. Sewage Enforcement Officer – Doug Duncan 724-679-4860.

2. Costs vary depending on the situation/testing needs.

(Note: the DEP planning review fee is a separate fee paid to DEP by the applicant.)

B. Excavation and water needed for testing are the responsibility of the applicant.

C. Pit & Perc Tests and ALTERNATE sites are required for each lot in a subdivision.

D. Non-building lots for AGRICULTURAL purposes may be permitted,

but a WAIVER must be signed.

(If an exemption from DEP planning review is being applied for, it must be approved and signed by the Township SEO & Secretary. Contact the SEO concerning eligibility for exemption.

SUBDIVISION/LAND DEVELOPMENT INSTRUCTIONS

A. DEP Subdivision Module can be completed and submitted to DEP.

**(must use most current form REV.10/2005 with Instruction for Completion of Component 1,

Exception to the Requirement to Revise the Official Plan. IF THE TOTAL of lots subdivided from 5/15/72 exceeds 10 - you cannot use this form. Contact DEP for the correct form. The Planning Modules have check-off lists of requirements.  The Module or Component must be reviewed and considered complete by the Sewage Enforcement Officer

B. DEP will advise by email of their decision.

III. Engineer Drawing or Survey

A. The Subdivision / Land Development Application has a check-off list of minimum requirements.

B. You and your engineer/surveyor are urged to work with the Township Planning Commission.

IV. Approval by Township Planning Commission, Board of Supervisors, and Butler County Planning Commission (ALSO see Major and Minor Land Development Procedures)

- The Township Planning Commission meets on the 3rd Thursday of each month if needed. Their meetings start at 6:00 pm at the Municipal Building located at 815 West Park Road, Slippery Rock, PA 16057

1. Subdivisions and Land Developments must be received by the Township Secretary at least 2 weeks prior to the Planning Meeting.

B.  Your plan must be submitted to Butler County Planning Commission. The County Planning Commission meets on the 3rd Wednesday of each month at 4:30 P.M. Subdivisions and/or Land Developments must be received in the Butler County Planning Commission office 7 days before the meeting in order to be placed on the agenda. County review and comment is required!!

Butler County Planning Commission has their own applications and fees.

2. A copy of the drawing will be retained by the county.

C. The Township Board of Supervisors meets on the first Tuesday of each month at 6:00 pm at the Municipal Building.

Final Approved Plan must be submitted on a Mylar signed and notarized by the property owners before the Supervisor meeting.

1. All specifications must be met, including final DEP approval.

2. Township Supervisors approval and signatures must be completed.

3. Township Planning Commission approval & signatures must be completed.

4. COUNTY Planning Commission approval & signatures are also required.

5. A copy of the drawing & related information will be retained by the Board.

D. ALL LAND DEVELOPMENT PLANS, including but not limited to Subdivisions, Lot-Line Revisions or Consolidations, or other Developments of Land (that are not necessarily a division of land) MUST BE RECORDED at the Butler County Recorder of Deeds office within ninety (90) days after all signatures are obtained (90 days from the last approval date, which is usually the Township Board of Supervisors.  Insofar as multiple-page land development plans, you must check with the Township Planning Commission to clarify which pages need to be recorded (including but not limited to the signature page and site plan with stormwater). Please check with the Recorder's office to get the current recording fee(s). The landowner/developer(s) or their engineers/surveyors are responsible for this recording and MUST submit a ‘Recorder’s Certificate (with the plan book and page numbers indicated) to the Township as proof of recording.

NO property transfers can be completed, and NO building or septic permits can be issued until such proof of recording is submitted. If plan(s) are not recorded within the 90-day window, said approval shall be deemed voided and rescinded and the plan must be resubmitted if approval is sought by the owner/developer. Paper copies of the plan(s) are acceptable by the County Recorder's office.

V. Permits: (After a Subdivision is approved & recorded and/or After a Land Development has been submitted, reviewed and approved.

I. A septic permit for a lot in the current subdivision can only be issued after the DEP sewage module has been approved by DEP <Dept. Of Environmental Protection.> (Obtain instructions from the Township SEO, Doug Duncan).

II. A building permit can only be issued after the sewage module has been approved by DEP (in the case of current subdivision or land development), and you have obtained the septic permit. Worth Township Building Permit application is available at the office and MDIA Application is on the township website under Permits.

III. A driveway permit is required if creating to either a township or State Road. Application and site distance requirements are on the township website under Permits.  If driveway is on a state highway you must obtain a driveway permit through PennDot.

IV. An “address” for new lots/houses is generally issued during the building permit process.

Contacts:

Township Secretary – Shari Kreutz 815 West Park Road Slippery Rock, PA 10657

Office Number724-368-3545 email info@worthtwp.org

Township Sewage Enforcement Officer – Doug Duncan, Primary 724-679-4860

Worth Township Planning Commission Chairman Ken Moniot – 724-679-5604

Driveway Permits (to access a Township Road) – Shari Kreutz

Driveway Permits on to a State road - PA Dept. of Transportation, Indiana, PA 724-357-2898

PA UCC BUILDING PERMITS & INSPECTIONS & BCO under PAUCC –

MDIA 101 N. Meadow Drive, Wexford, PA 15090.  Harry Goerl 1-800-732-6342

*Township Engineer – EADS Group 15392 Route 322 Clarion, PA 16214

1-814-764-5050

*Dept. of Environmental Protection (Tomisa Schmeider) 121 N. Mill St., New Castle, PA 16101 724-656-3160

*Butler Co. Recorder of Deeds Office - P.O. Box 1208, Butler, Pa 16003-1208 724-284-5340